Confidential


ISO INVENTION DISCLOSURE FORM
CONFIDENTIAL and ATTORNEY-CLIENT PRIVILEGED COMMUNICATION

Rev. 1 – October 2003

DATE: Apr/30/2007
It is important to provide accurate and detailed information on this form.  The information will be used to evaluate your invention for possible filing as a patent application.  Send Invention Disclosure forms electronically to the ISO eRoom.
	Last Name:  Kau
	First Name: DerChang
	M.I.

	Business Phone Number: +1-408-765-0266
	Business Fax Number: +1-408-765-5775
	Mailstop: RN3-01

	E-mail address: DerChang.Kau@intel.com
	WWID: 10077599

	Citizenship: USA
	
	
	

	Home Address: 10847 Linda Vista Drive

	City: Cupertino
	State: CA
	Zip: 95014
	Country: USA

	Business Address: 2200 Mission College blvd
	
	

	City: Santa Clara
	State: CA
	Zip: 95054
	Country:


	Last Name:  Fackenthal
	First Name: Richard
	M.I. E

	Business Phone Number: +1-916-356-2291
	Business Fax Number: 
	Mailstop: FM3-174

	E-mail address: Rich.Fackenthal@intel.com
	WWID: 10061609

	Citizenship: USA
	
	
	

	Home Address:

	City: 
	State: CA
	Zip: 
	Country: USA

	Business Address: 
	
	

	City: 
	State: CA
	Zip: 
	Country:


	Last Name:  Bedeschi
	First Name: Ferdinando
	M.I.

	Business Phone Number: 
	Business Fax Number: 
	Mailstop:

	E-mail address: ferdinando.bedeschi@st.com 
	WWID:

	Citizenship: 
	
	
	

	Home Address: 

	City: 
	State: 
	Zip: 
	Country: 

	Business Address: 
	
	

	City: 
	State: 
	Zip: 
	Country:


	2.  Title of Invention:

Four Vertically Stack Layers of OUMS Memory Array


	3.  What technology/product/process (code name) does your invention relate to (be specific if you can):

A path finding test vehicle for OUMS Memory Array validation and exploration using P894 Process Technology and beyond 


	4.
Include several key words to describe the technology area of the invention in addition to # 3 above:

OUM, OUMS, Phase Change Memory, PCM, Triggered PCM, P-RAM


	5.
Stage of development (i.e. % complete, simulations done, test chips if any, etc.):
Path Finding: Concept exploration.  


	6a.  Has a description of your invention been (or planned to be) published outside of the ISO Collaboration:

If YES, please identify the publication and the date published:
No


	6b.  Has your invention been used/sold or planned to be used/sold by Intel, ST Micro, Ovonyx, or others?

No
If YES, date it was sold or will be sold:


	6d.  If the invention is embodied in a semiconductor device, what is the actual or anticipated date of tapeout?

Not Planned Yet.


	6e.  If the invention is software, actual or anticipated date of any beta tests or other public distribution outside the ISO Collaboration: N/A


	7.  Is this invention related to any other invention disclosure that you have recently submitted?  If so, please give the title and

     inventors: No



PLEASE READ AND FOLLOW THE DIRECTIONS ON

HOW TO WRITE A DESCRIPTION OF YOUR INVENTION

Please provide a description of the invention and include the following information:

1. Explain the problem being addressed by the invention.
Reduce effective cell size of a bit of memory (0/1)
[image: image7.emf]k


l


2


×


M


AE


×






2



MAE




Effective cell sizes is measured by:             , where
· :  Planar Cell layout:  
· :  Minimum Pitch Capability for technology 

· M: Electrically multilevel or physically multilayer (bits/cell-area)

· AE: Array efficiency; Periphery analog, digital, I/O circuits for memory array operations
The following table benchmarks memory cell size performance between current art and this disclosure


[image: image1.emf]  DRAM NAND

MOS

PCM

BJT/Diode

PCM

OUMS

(estimated)

This 

Disclsosure

Layout: 

k

6~8 4 5~10 4~6 4 4

MLC: 

M

1 2 (Elec) 1.5 (Elec) 1.5 (Elec) 2 (Phy) 4 (Phy)

AE 70% 50% 50% 40% 70% 60%

Eff. Cell Sizes ~10

l

² ~4

l

² ~12

l

² ~8

l

² ~3

l

² < 2

l

²



This invention disclosure addresses the reduction of effective cell sizes without performance penalty.
2. Identify the closest or most pertinent prior art that you are aware of.

· What is done today to alleviate the problem solved by the invention?
“Effective cell size” reduction of a given technology node comes from cell layout, MLC capability and Arrary efficiency.  OUMS technology gives the best in the class.  Current art, proposed by Bedeschi et.al., deploys 2 layers of memory stacking on the top of transistor which is used for array decoding, programming and sensing operation.  3 embodiments were proposed, shown in Fig 1.

[image: image8.emf]1

3

2

4


[image: image2]
3. Describe in detail what the components of the invention are and how the invention works.

· Provide a summary paragraph of what your invention is, and what it does.
· Next, briefly describe each of the key elements, and how they relate to each other in the overall invention (optional: reference drawings)
The invention disclosure enabled 4 layers of memory stack without increasing circuit complexity therefore maintaining array efficiency.  
It use the layout of 4 with tiles common-centroid symmetry as shown in Fig.2.   The vertically and horizontally interleaved decoding, selecting, sensing and programming circuits are placed directly under each of the tile, exhibited in Fig.2
[image: image9.emf]3

4

1 2


[image: image3]
For each tile there are 4 unique decoding supported by the circuit below the memory tiles, as shown in Fig 3.

[image: image4]
The integration of the common-centroid symmetry tiles forming a memory array is exhibited in Fig 4.


[image: image5]
Three variants of 4-Layer memory stacks with circuit under the array tiles are illustrated using 3D cross sections.  As shown in Fig. 5, it exhibits the connectivity of 4 layers of OUMS memory to the decoding circuits.

[image: image6]
4. Describe advantage(s) of your invention over what is currently being done.
Smallest effective-cell size enabled.  
5. Include at least one figure illustrating the invention.  If the invention relates to 


software, include a flowchart or pseudo-code representation of the algorithm.

6. How would infringement be detected?
· Identify how use of the invention would be visible, measurable, provable
· Examples of Detection Methods: Visual Examination, Reading Product literature, Reading Programming Reference Manuals, Reading Data Sheets, Reverse Engineering
Cross section on memory layer and decoder connectivity
DATE:    Apr/30/2007
INVENTOR:   DerChang Kau


DATE:    Apr/30/2007
INVENTOR:   Rich Fackenthal


DATE:    Apr/30/2007
INVENTOR:   Ferdinando Bedeschi


OPTIONAL WITNESS SIGNATURE

DATE:    


WITNESS:   



BY THIS SIGNING, I (WITNESS) ACKNOWLEDGE THAT I HAVE READ AND UNDERSTAND THIS 

DISCLOSURE.
(a)





Fig.1 2 layers memory stack on the top of wordline and bitline circuit 


(a).   common-centroid symmetry with 4 tiles of bi-layer memory implementation.


(b)    1 tiles of bi-layer memory implementation.


(c)    Vertically interleaved 2 tiles of bi-layer memory implementation 





Fig 5.  3 variants of 4-layer stack OUMS memory array.  This invention disclosure is also applicable to other stackable cross point memory array




















(b)





(c)





Fig 2.  4   symmetric tiles over decoding, sensing and programming circuits





Fig 3.  Decoding topology provided by the common-centroid symmetry topology 





(a) 





(b) 





(c) 





Fig 4.  Decoding topology provided by the common-centroid symmetry topology 








[image: image10.emf][image: image11.emf]Y-Path

X-Path

X-Path

Y-Path

X-Path

X-Path

Y-Path Y-Path

West

East

East

West

North

South

North

South

Tile 1 Tile 2

Tile 3

Tile 4

[image: image12.emf]tile with 2 X-decoders under

West Side of 

decoder

East Side of 

Decoders

[image: image13.emf]tile with 2 Y-decoders under

North side

of select

South side 

of select

[image: image14.emf]4 unique address is provided 

By 4 sides of dec/sel topology 

W

E

N

S

[image: image15.emf][image: image16.emf]1 



² Cell 

4 Layers of Memories

Row

Column

OUMS

Color and Legend

Row

Column

OUMS

Color and Legend

E-1

S-1

E-2

W-3

N-2

SE

SE

SE

NE

NE

NE

S-3

W-4

N-4

NW

NW

NW

SW

SW

SW

E-1

S-1

E-2

W-3

N-2

SE

SE

SE

NE

NE

NE

S-3

W-4

N-4

NW

NW

NW

SW

SW

SW

[image: image17.emf]1 



² Cell

2 layers of Bi-Layer memories

E-1

S

Row

Always

W-2

E-3

Column

Always

N

W-4

SE

SE

SE

SW

SW

SW

NE

NE

NE

NW

NW

NW

E-1

S

Row

Always

W-2

E-3

Column

Always

N

W-4

SE

SE

SE

SW

SW

SW

NE

NE

NE

NW

NW

NW

South & North Pitch Cell 

will alternate between 

Column & Row functions

[image: image18.emf]1 



² Cell

4 Symmetrical Memory Layers

• Either All levels of pitch cells alternate between Column & Row

• Or Memory Cells (storage and selector) are symmetric (Heater-less)

E-0

S-1

W-2

N-3

E-4

SE

SE

SE

SW

SW

SW

NW

NW

NW

NE

NE

NE

E-0

S-1

W-2

N-3

E-4

SE

SE

SE

SW

SW

SW

NW

NW

NW

NE

NE

NE

_1239435678.xls
Sheet1

				DRAM		NAND		MOS
PCM		BJT/Diode
PCM		OUMS
(estimated)		This 
Disclsosure

		Layout: k		6~8		4		5~10		4~6		4		4

		MLC: M		1		2 (Elec)		1.5 (Elec)		1.5 (Elec)		2 (Phy)		4 (Phy)

		AE		70%		50%		50%		40%		70%		60%

		Eff. Cell Sizes		~10l²		~4l²		~12l²		~8l²		~3l²		< 2l²





Sheet2

		





Sheet3

		






