Confidential


ISO INVENTION DISCLOSURE FORM
CONFIDENTIAL and ATTORNEY-CLIENT PRIVILEGED COMMUNICATION

Rev. 1 – October 2003

DATE: Sep/8/2008

It is important to provide accurate and detailed information on this form.  The information will be used to evaluate your invention for possible filing as a patent application.  Send Invention Disclosure forms electronically to the ISO eRoom.
	Last Name:  Kalb
	First Name: Johannes
	M.I. A

	Business Phone Number: +1-408-765-2077
	Business Fax Number: +1-408-765-5775
	Mailstop: RN3-01

	E-mail address: John.a.Kalb@intel.com
	WWID: 11227641

	Citizenship: Germany
	
	
	

	Home Address: 1330 The Alameda, Apt. 379

	City: San Jose
	State: CA
	Zip:95126
	Country: USA

	Business Address: 2200 Mission College blvd
	
	

	City: Santa Clara
	State: CA
	Zip: 95054
	Country: USA


	Last Name:  Kau
	First Name: DerChang
	M.I.

	Business Phone Number: +1-408-765-0266
	Business Fax Number: +1-408-765-5775
	Mailstop: RN3-01

	E-mail address: DerChang.Kau@intel.com
	WWID: 10077599

	Citizenship: USA
	
	
	

	Home Address: 10847 Linda Vista Drive

	City: Cupertino
	State: CA
	Zip: 95014
	Country: USA

	Business Address: 2200 Mission College blvd
	
	

	City: Santa Clara
	State: CA
	Zip: 95054
	Country: USA


	2.  Title of Invention: 
READING MULTI-LEVEL OUM MEMORY


	3.  What technology/product/process (code name) does your invention relate to (be specific if you can):

A path finding test vehicle for OUMS Memory Array validation and exploration using P894 Process Technology and beyond 


	4.
Include several key words to describe the technology area of the invention in addition to # 3 above:

OUM, OUMS, Phase Change Memory, PCM, Triggered PCM, P-RAM


	5.
Stage of development (i.e. % complete, simulations done, test chips if any, etc.):
Path Finding: Silicon learning and validation is obtained.   64Mb test chip, aka PT180, will implement the disclosed method. 


	6a.  Has a description of your invention been (or planned to be) published outside of the ISO Collaboration:

If YES, please identify the publication and the date published:
No


	6b.  Has your invention been used/sold or planned to be used/sold by Intel, ST Micro, Ovonyx, or others?

No?
If YES, date it was sold or will be sold:


	6d.  If the invention is embodied in a semiconductor device, what is the actual or anticipated date of tapeout?

Q3, 2007


	6e.  If the invention is software, actual or anticipated date of any beta tests or other public distribution outside the ISO Collaboration: N/A


	7.  Is this invention related to any other invention disclosure that you have recently submitted?  If so, please give the title and

     inventors: No



PLEASE READ AND FOLLOW THE DIRECTIONS ON

HOW TO WRITE A DESCRIPTION OF YOUR INVENTION

Please provide a description of the invention and include the following information:

1. Explain the problem being addressed by the invention.
Among all the read schemes in an OUMS array, only two states, SET and RESET, can be distinguished.  The threshold voltage detection for multi-level threshold placement of RESET states is deemed impractical due to Ovnoics threshold switching effect and the corresponding snapback induced disturbances to a RESET state. 

2. Identify the closest or most pertinent prior art that you are aware of.

· What is done today to alleviate the problem solved by the invention?
Savransky et al disclosed a ramp detection scheme of reading OUM threshold voltage.   In the disclosure, it depends on the non-destructive READ of OUM characteristics by clamping the path current close to yet higher than OUM threshold current.   By reading the voltage at such current, device threshold voltage is detected without disturbing a RESET state of OUM cell.

The practicality and reliability of non-destructive READ of OUM RESET state under near threshold current is virtually nonexistence.

3. Describe in detail what the components of the invention are and how the invention works.

· Provide a summary paragraph of what your invention is, and what it does.
· Next, briefly describe each of the key elements, and how they relate to each other in the overall invention (optional: reference drawings)
Using bias and temperature dependent RESET disturbance characteristics of OUM, this invention disclosure includes 4 components.
1. Threshold with current clamp before READ sensing to eliminate drift characteristics of glassy materials 
2. Voltage sensing method by detecting cell potential drop at READ current near threshold voltage

3. Clamping path current below a maximum allowable level before refresh
4. Content refresh cycle based on time to disturbance per above READ conditions
4. Describe advantage(s) of your invention over what is currently being done.


5. Include at least one figure illustrating the invention.  If the invention relates to 


software, include a flowchart or pseudo-code representation of the algorithm.
6. How would infringement be detected?
· Identify how use of the invention would be visible, measurable, provable
· Examples of Detection Methods: Visual Examination, Reading Product literature, Reading Programming Reference Manuals, Reading Data Sheets, Reverse Engineering
DATE:    


INVENTOR:   



DATE:    


INVENTOR:   



DATE:    


INVENTOR:   



OPTIONAL WITNESS SIGNATURE

DATE:    


WITNESS:   



BY THIS SIGNING, I (WITNESS) ACKNOWLEDGE THAT I HAVE READ AND UNDERSTAND THIS 

DISCLOSURE.
