Contains or Derived from Supplier Confidential Information

If you do not have a business need to receive this information (e.g. you may have switched jobs from GEMS to another BU), you should not be receiving the information and may have received this email inadvertently. Please immediately notify the email author to rectify the distribution list.

Logic Technology

Si Technology Roadmap & Product Planning Alignment – Follow-up meeting with IDC’s additional request on additional cells and “.LIB” for PrimeTime.  All 16 requested extraction-ready cells created for both N2 reference and N2-SPR.  Si Smart methodology using SPX/RFX testcase with PDK compact model successfully created “.LIB.”   Met with GFX team and introduced our group and capability to deliver pre-PDK collateral.  GFX team appears interested to do L1 type of metric, bringing it to Satish for feedback.  Atom team meeting replanned to WW13.1.

SF2 & Golden Gate Creek (GGC) –   SF2 V0.9DK collateral download remains in progress (~90% complete) with expectation of installation in the FTE environment for PPA analysis by WW13.1.  Download of additional IP (GPIO, Promise, PLL) expected in WW13.

Pre-PDK Mockup & Methodology Development – Complete TCAD model calibration for SRAM HD PD and PU device characteristics. Continuous to improve the TCAD variation model for SRAM HD device.  Mock-L2 methodology development, ongoing including RC extraction segmentation for NXTGRD,  debugging vias errors with addition “My” layer.  SLiC M2 track mapping issue resolved, generating a mock N2 library can now proceed.

Si Process Costing Model – TSMC Mock A14 v0.01 completed and released for costing.  Structural cost increases due to moving all well-based masks from 193 to 193i, addition of 3 EUV layers due to M2 track scaling and M0 pitch scaling requires another 35P layer and a 2nd VD respectively.  Additional sheet cut mask required to support HRO cells with very large OD.

Memory Technology

SF4 Muddy Creek/25um µB FO-3D @ SPIL – Test program validation is ongoing and tracking one week ahead.  All test items have been implemented and checked out and functional.  Issues outstanding, MBIST@speed and TE@speed, are being segmented. First full-wafer run is planned for next week.

3DIC Enabling

Cool Creek/15um µB CoW @ ASE – Experimental trial #1 is ongoing. Wafer ship-out trending for April 8 (delayed by approx 10 days, from end-of-March). Preparing new mask set for Trial #2.

SF4 Tower Creek/25µm µB WoW @ SF – On track for May tape-In.  Tape In checklist review and DRC/DFM waiver review with Samsung to start next week.  Updated MIMCAP insertion flow resulted in a large number of DRC; review in progress with Samsung; quick resolution with Samsung is anticipated.  Full chip integration in progress with no known issues.  RTL is considered frozen for Tape In.

Mixed Signal IP/Collaterals Design

N2P eFuse HIP – Design and layout progress is on-track. 85% pre-sim progress and related schematics are checked-in.  Meet voltage detection spec (.63v~.96v) across pvt and match power consumption with N3.