Contains or Derived from Supplier Confidential Information

If you do not have a business need to receive this information (e.g. you may have switched jobs from GEMS to another BU), you should not be receiving the information and may have received this email inadvertently. Please immediately notify the email author to rectify the distribution list.

Executive Summary

= Mock A10 NW-based design rule and H080 HD library template cell complete.

= SF2/PBK L2 assessment completed yielding 2~3% L2 improvement.

+ Complete Terraced Memory Cube Q2 milestone delivering significant lower cost and complexity of making than HBM.

Logic Technology

TSMC Si Technology Roadmap & Product Planning Alignment – Two NW options are under consideration, mA10_V04 H080-42 and 2 mA10_V05 H076-42.  Mock A10_V04 NW-based design rules complete and H080 HD library template cell complete.  In the meantime, team is working thru technical realignment of UC consolidation and GPU expansion.

Samsung SF2 – 168H based L2 assessment completed.  2~3% performance improvement exhibited with performance boosting cells.  No change to SF engagement.

Pre-PDK Mockup & Methodology Development – SLiC generated dotLEF for mN2 first run successful through full APR flow. 25% performance gap is shown.  Gap mitigation strategy to mock A14 is under development.   Primary direction is expanding/improving cell library.  However, two months schedule impact or push-out is expected.

Si Process Costing – SF4 process FEOL, MEOL and Mx flows complete.  Updated 1276 flow convert SAQP fin backbone to EUV mask, and 1278 convert 2xArFi+shrink to EUV mask for GCN – EUV initially driven by cost, but seems like it is for process simplicity

Memory Technology

SF4 Muddy Creek/25um µB FO-3D @ SPIL – Stack die test development are on track.  Assembly timeline continues to track for power-up in WW31 (first batch) and WW32 (second batch). Test program development and preparation is ongoing, with focus on implementation of the bump defect detection and repairs.

Terraced Memory Cube (TMC) – Good progress in TMC.   Complete Q2 milestone on time.  TMC architecture and BKM process Flow yields > 40% lower cost than HBM.  Startup Component Module target Spec (MTS) defined.  Creative modular integration scheme reduces silicon complexity.   Only 2 chiplets (vs. 9 chiplets in HBM) are needed for 16 tiers 3D stack.  Innovated chiplet thickness design enables more than 50% thermal resistance reduction than HBM.  SI/PI and thermal flow improvement are in high gear. Full review to be presented in next week’s PF project review meeting.

3DIC Enabling

Cool Creek/15um µB CoW @ ASE – All activities are on track or ahead of schedule.   Full-loop wafers shipped from Samsung and will be ready for us to start SORT testing next week.