Subcommittee:		MT

Publicity Focal Point:	Zhiqiang Wei, Avalanche

Phone: +1-9253534239

Email: zhiqiang.wei@avalanche-technology.com

Focal Point Back-up:	Hongmei Wang

Phone: 208-7944511

Email: hwang@micron.com

Paper#:	139

Title:	Oxide-semiconductor Channel Transistor DRAM(OCTRAM) with 4F2 Architecture

Authors:	Shosuke Fujii

Affiliation:	Kioxia Corporation, Japan

A New 4F2 DRAM Technology

The OCTRAM, utilizing InGaZnO vertical channel transistors (VCTs), represents a breakthrough in achieving 4F2 DRAM architectures. The integration of gate-all-around InGaZnO VCTs has been integrated on the top of high aspect ratio capacitors, which allows for high ON current (>15µA/cell) and ultra-low leakage (<1aA/cell), providing stability against BEOL processing. A 275Mbit OCTRAM array has been built to demonstrate the potential for high-density, low-power DRAM applications. At IEDM, the Kioxia and Nanya team will address the technological advancements and future implications of this innovative DRAM architecture.